Module 1: Electrical Raceways Information Sheet 1.1: Types of Conduits

# Learning Objectives

Upon completion of this information sheet, the learner must be able to:

- Identify the four major types of conduits (RSC, IMC, EMT, and PVC) based on physical appearance and weight.

- Determine the correct application for each conduit type according to the Philippine Electrical Code (PEC) standards.

- Differentiate between joining methods (threading vs. non-threading) and the tools required for each.

- Recall the standard trade measurements and lengths used in the Philippine electrical industry.

# 📝 Introduction

In any electrical installation, the wires and cables are the "veins" of the system, carrying the vital power needed for equipment. However, these wires are fragile; they can be cut, chewed by rodents, or degraded by sunlight and water.

Conduits (also known as raceways) act as the "armor" for these conductors. Choosing the wrong type of conduit isn't just a technical error—it's a safety hazard. A conduit that is too thin in a high-traffic factory might get crushed, leading to short circuits or fires. Conversely, using heavy-duty steel where lightweight plastic is sufficient increases costs unnecessarily.

This lesson covers the four primary conduits recognized by the Philippine Electrical Code (PEC). Mastering these materials is the first step toward becoming a professional electrician who ensures safety, durability, and code compliance in every project.

# Detailed Conduit Breakdown

## 1. Rigid Steel Conduit (RSC)

The "Heavyweight Champion": This is the toughest raceway available.

Detailed Description: RSC is hot-dip galvanized both inside and out. The interior is specifically designed to be smooth (a "smooth raceway") to prevent the wire insulation from stripping when pulled through long runs.

Measurements & Identifying Marks:

- Wall Thickness: Approximately 2.64mm to 6.67mm

- Color Code: Often identified by a Red label.

- Threading: Requires a pipe threader; it uses NPT (National Pipe Thread).

## 2. Intermediate Metal Conduit (IMC)

The "Modern Hybrid": Thinner than RSC but stronger than EMT.

Detailed Description: It provides almost the same level of protection as RSC but is lighter and easier to work with. It is coated with a zinc-based finish for rust resistance.

Measurements & Identifying Marks:

- Wall Thickness: Roughly 30% thinner than RSC.

- Color Code: Often identified by an Orange or Yellow label.

## 3. Electrical Metallic Tubing (EMT)

The "Thin-Wall": The standard for indoor commercial wiring.

Detailed Description: Because it is thin, it cannot be threaded. It is joined using set-screw or compression fittings. It is very easy to bend with a hand bender, making it perfect for complex indoor routes.

Measurements & Identifying Marks:

- Wall Thickness: Thinnest metal option.

- Identification: Usually has a Green or Silver label.

## 4. Rigid Non-Metallic Conduit (PVC)

The "Corrosion-Proof" Solution: Best for wet or underground environments.

Detailed Description: Made of Polyvinyl Chloride. It is non-conductive (won't carry electricity if a wire shorts to it) and completely rust-proof.

Measurements & Identifying Marks:

- Schedules: Schedule 40 (Standard) and Schedule 80 (Extra Heavy Wall).

- Joining: Uses chemical solvent cement (glue) to create waterproof seals.

# Quick Reference Table

| Type | Material | Joint Method | Best For... |
| --- | --- | --- | --- |
| RSC | Heavy Steel | Threaded | Areas with high risk of physical impact |
| IMC | Medium Steel | Threaded | General outdoor/industrial use |
| EMT | Light Steel | Set-screw/Compression | Indoor walls and ceilings |
| PVC | Plastic | Solvent Cement | Underground or wet/salty air |