Module 1: Electrical Raceways Self-Check 1.2: Conduit Installation Principles

Direction: Select the best answer for each question. Write the letter of your answer on the line provided.

| 1. ______ | What is the maximum total number of degrees of bends allowed between pull points (e.g., junction boxes) according to the PEC? |
| --- | --- |

A. 180°

B. 270°

C. 360°

D. 540°

| 2. ______ | When interpreting an electrical drawing, what does a dashed line typically represent? |
| --- | --- |

A. Ceiling-mounted conduit

B. Exposed wall-mounted conduit

C. Underground or under-floor conduit

D. A telephone line

| 3. ______ | If you are using a 1/2" EMT bender, what is the standard Take-Up value for a 90° stub-up? |
| --- | --- |

A. 5 inches (12.7 cm)

B. 6 inches (15.2 cm)

C. 8 inches (20.3 cm)

D. 10 inches (25.4 cm)

| 4. ______ | Why is it critical to ream the ends of a conduit after cutting? |
| --- | --- |

A. To make the conduit look professional

B. To remove sharp burrs that can slice wire insulation

C. To make the conduit lighter

D. To allow the pipe to bend easier

| 5. ______ | What is the multiplier used to calculate the distance between marks for a 30° offset bend? |
| --- | --- |

A. 1.4

B. 2.0

C. 2.6

D. 6.0

| 6. ______ | According to PEC standards, conduit must generally be supported within how many meters of every outlet box or fitting? |
| --- | --- |

A. 0.3 meters

B. 0.9 meters

C. 1.5 meters

D. 3.0 meters

| 7. ______ | Which tool is used to ensure that a vertical conduit run is perfectly straight up and down? |
| --- | --- |

A. Scale Ruler

B. Tape Measure

C. Plumb Bob or Spirit Level

D. Conduit Reamer

| 8. ______ | In an offset bend, what happens to the total length of the conduit run? |
| --- | --- |

A. It gets longer

B. It stays exactly the same

C. It "shrinks" (gets shorter)

D. It doubles in weight

| 9. ______ | Where would you find the specific conduit and wire sizes required for a project on an electrical plan? |
| --- | --- |

A. Legend

B. Floor Plan

C. Schedule of Loads

D. Title Block

| 10. ______ | What is the result of a bend that is "kinked" or flattened? |
| --- | --- |

A. It is easier to pull wires through

B. It is a code violation and makes wire pulling difficult/impossible

C. It increases the voltage of the circuit

D. It makes the conduit more durable

— End of Self-Check 1.2 —